SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I Peer mohammed (192312666L)  certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

I peer mohammed

Annexure A

Short Answer Test

1. A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

Ans : Given

- File size = 150 MB

- Bandwidth = 50 Mbps

- Segment size = 1500 bytes

- Data Link overhead = 40 bytes/frame

Step 1: File size in bytes

150×106=150,000,000 bytes

Step 2: Number of segments

150,000,0001500=100,000 segments

Each segment becomes one frame.

Frames = 100,000

Step 3: Total Data Link overhead

100,000×40=4,000,000 bytes=4 MB

Step 4: Total transmitted data

150+4=154 MB

Step 5: Transmission time

Convert to bits:

154×8=1232 Mb123250=24.64 seconds

Answer

- Segments = 100,000

- Frames = 100,000

- Overhead = 4 MB

- Transmission time = 24.64 s

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Ans :

- Given

- Window size = 6

- Total frames = 48

- Number of windows

486=8

- Retransmissions

One frame lost per window.

8×1=8

- Answer

- Windows = 8

- Retransmissions = 8

- Flow Control

Flow control prevents the sender from overwhelming the receiver, improving efficiency and avoiding buffer overflow.

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

Ans: Given

- Packet = 12,000 bytes

- MTU = 1500 bytes

- Header = 20 bytes

Maximum payload per fragment

1500−20=1480 bytes

Fragments

120001480=8.1

Hence

9 fragments

Header overhead

9×20=180 bytes

Answer

- Fragments = 9

- Header overhead = 180 bytes

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.
Ans:

- Given

- Window size = 6

- Total frames = 48

- Number of windows

486=8

- Retransmissions

One frame lost per window.

8×1=8

- Answer

- Windows = 8

- Retransmissions = 8

5. A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.	
Ans:

- Given

- File = 600 MB

- Checkpoint every 60 MB

- Failure after 390 MB

- Checkpoints

At

60,120,180,240,300,360

Total

6 checkpoints

- Data retransmitted

Last checkpoint = 360 MB

Failure = 390 MB

390−360=30 MB

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

Ans:

Given

- Original = 900 MB

- Compression = 40%

- Encryption overhead = 5%

Compressed size

900×0.60=540 MB

Encrypted size

540×1.05=567 MB

Reduction

900−567900×100=37%

Answer

- Compressed size = 540 MB

- Encrypted size = 567 MB

- Reduction = 37%

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

### Ans: Given

- File = 3 GB = 3000 MB

- Throughput = 120 Mbps

- Request size = 3 MB

- Requests

30003=1000

- Transmission time

Convert file into megabits

3000×8=24000 Mb24000120=200 seconds

- Answer

- Requests = 1000

- Time = 200 seconds

8.A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

Ans : Processing delay

4+3+5+2+1=15 ms

Total delay

15+18+12=45 ms

Answer

- Total end-to-end delay = 45 ms

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

Ans :

- Given

- Useful data = 80 MB

- Frame size = 1600 bytes

- Overhead = 80 bytes

Useful payload/frame

1600−80=1520 bytes

- Frames

80,000,0001520=52,632 frames

- Total overhead

52,632×80=4,210,560 bytes

≈ 4.21 MB

- Efficiency

15201600×100=95%

- Answer

- Frames = 52,632

- Overhead = 4,210,560 bytes (≈4.21 MB)

- Efficiency = 95%

10.A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.		
Ans :

- Given

- Original = 240 MB

- Compression = 30%

- Segment size = 1400 bytes

- Data Link overhead = 60 bytes

- Compressed size

240×0.70=168 MB

Convert to bytes

168,000,000 bytes

- Segments

168,000,0001400=120,000

- Protocol overhead

120,000×60=7,200,000 bytes=7.2 MB

- Final transmitted data

168+7.2=175.2 MB

- Answer

- Compressed size = 168 MB

- Segments = 120,000

- Protocol overhead = 7.2 MB

- Final transmitted data = 175.2 MB